User

Property Managers

Cleaners

Functional Requirement

Log In

User: Property Managers

Assigned username and password, with API token

No need to register for now

Destination: Overview

Log Out

User: Property Managers

Destination: Log In page

Left Sidebar

Overview

Floor Plan

--------

Log Out

--------

Notification

Overview

User: Property Managers

Basic Information of the Property

static

Notification

- Real time

- If everything is fine, display “Washroom situation is excellent” / “洗手間整體情況極佳”

- If not, display the notification messages.

- Can be scrolled down to display many notification messages.

Service Down Time

- Real time

- The total downtime of all the utensils due to late change of utensils.

- If the downtime of more than one utensil is overlapped, it should be counted separately.

- Calculate the total downtime of this week (from Monday) and this month (from 1st of this month).

Usage Rate

- Real time

- Categorized by the floor where toilets are located

- Hover: show “no. of occupied toilets / total no. of toilets, XX%”

- The color of the bar

| Color | Red | Yellow | Green |
| --- | --- | --- | --- |
| Percentage | =100% | 75 -99% | 0-74% |

People Count

- Real time

- This number is the total people count of no. of people that used the toilets within the day (from the opening time of the property).

Overall Indicator

- Real time

- The overall conditions of the above five stuff in the whole property/shopping mall/building.

- The bigger the pentagon, the better.

- Calculation Method:

  - Toilet Paper Score:

Total No. of toilet paper rolls with more than 25% toilet paperTotal No. of toilet paper sensors in the property

  - Soap Dispenser Score:

Total No. of soap dispensers with more than 25% soapTotal No. of soap dispenser sensors in the property

  - Rubbish Bin Score:

Total No. of rubbish bins with less than 75% rubbishTotal No. of rubbish bin sensors in the property

  - IAQ Score:

Total No. of IAQ sensors with all 5 paprameters are excellentTotal No. of IAQ sensors in the property

  - Water Leakage Score:

Total No. of water leakage sensors with no leakage detectedTotal No. of water leakage sensors in the property

Toilets

If not applied, the column will be in grey color.

Name of toilets

- Static

- The names will be assigned by customers

- Categorized by the floor where toilets are located

People Count

- Real time

- Total people count of no. of people that used the specific toilet within the day

Usage Rate

- Real time

- Current usage rate of the specific toilet

- Hover: show “no. of occupied toilets / total no. of toilets, XX%”

- The color of the bar

| Color | Red | Yellow | Green |
| --- | --- | --- | --- |
| Percentage | =100% | 75 -99% | 0-74% |

Toilet Paper

- Real time

- Whether toilet paper rolls in each toilet cubicles are enough

- The color, number and hover of the circle

| Color | Red | Yellow | Green |
| --- | --- | --- | --- |
| Situation | At least one cubicle is out of toilet paper (threshold may be changed by customers) | The toilet paper in at least one cubicle has below 25% left (threshold may be changed by customers) | Other situations |
| Show No. of Cubicles with the above situation | Yes | No | No |
| Hover | Shows“Cubicle 1, 2, 3… is out of toilet paper” | Shows “Cubicle 1, 2, 3… is almost out of toilet paper” | Nothing |

Soap Dispenser

- Real time

- Whether soap dispensers near the water tanks are enough

- The color, number and hover of the circle

| Color | Red | Yellow | Green |
| --- | --- | --- | --- |
| Situation | At least one soap dispenser is out of soap (threshold may be changed by customers) | At least one soap dispenser has below 25% of soap left (threshold may be changed by customers) | Other situations |
| Show No. of Parameters with the above situation | Yes | No | No |
| Hover | “Temperature Ammonia is poor” | Shows “Soap Dispenser 1, 2, 3… is almost out of soap” | Nothing |

Rubbish Bin

- Real time

- Whether rubbish bins in the toilet are full

- The color, number and hover of the circle

| Color | Red | Yellow | Green |
| --- | --- | --- | --- |
| Situation | At least one rubbish bin is full (threshold may be changed by customers) | At least one rubbish bin is 75% full (threshold may be changed by customers) | Other situations |
| Show No. of Cubicles with the above situation | Yes | No | No |
| Hover | Shows“Rubbish Bin 1, 2, 3… is full” | Shows “Rubbish Bin 1, 2, 3… is almost full” | Nothing |

IAQ

- Real time

- Whether the Air Quality in the toilet is good

- Five parameters:

  - Temperature

  - Humidity

  - Hydrogen Sulphide

  - Ammonia

  - PM2.5

- The color, number and hover of the circle

| Color | Red | Yellow | Green |
| --- | --- | --- | --- |
| Situation | At least one parameter is poor/較差 (threshold may be changed by customers) | At least one parameter is average/良好 (threshold may be changed by customers) | All 5 parameters are excellent/極佳 |
| Show No. of Cubicles with the above situation | Yes | No | No |
| Hover | Shows “ Temperature: xx ºC Humidity: xx % H2S: excellent NH3: poor PM2.5: average ” (depends on real time data) | Shows “ Temperature: xx ºC Humidity: xx % H2S: excellent NH3: excellent PM2.5: average ” (depends on real time data) | Shows “ Temperature: xx ºC Humidity: xx % H2S: excellent NH3: excellent PM2.5: excellent ” (depends on real time data) |

Map

Click the map ICON to open the floor plan of the specific toilet

- The sensor status shown in the floor plan are updated real time

- This pop-up page will show the status of all the sensors

- Floor plan is generated / re-drawn according to the floor plans of the property. (see if we can develop an easier or automatic or non-code way to draw this floor plan for easier implementation in the future)

Floor Plan

IAQ

Click the  logo to open IAQ panel.

Click the  logo to open the explanation of each parameter.

| Parameter | Poor | Average | Excellent |
| --- | --- | --- | --- |
| H2S | (0.003,+∞) | (0.001, 0.003] | [0, 0.001] |
| NH3 | (0.015, +∞) | (0.01, 0.015] | [0, 0.01] |
| PM2.5 | (65 ,+∞) | (15, 65] | [0, 15] |
| Temperature | (-∞,20] or (29, +∞) | (20,22] or (27,29] | (22, 27] |
| Humidity | (90%, +∞) | (80%, 90%] | [0, 80%] |

Notification（still waiting for UI design）

- Show time occurred for each notification

- Group notifications by each toilet

- Sound Alert

- Send SMS to certain phone numbers